Highways and Transportation

### Network Management

Section 50

New Roads and Street Works Act 1991

Installation of Private Apparatus in the Highway

(including guidance on the required procedure)

Updated – July 2025

1	INTRODUCTION

1.1	Anyone wishing to install any apparatus either above or below ground in the publicly maintained highway (the Applicant) must apply for permission from Leeds City Council (the Council) under Section 50 and Schedule 3 of the New Roads and Street Works Act 1991 (NRSWA). The term ‘apparatus’ includes drains, cables, ducts, sewer pipes, water and gas pipes etc.

1.2	A person granted a Street Works Licence (the Licensee) becomes an undertaker for the purposes of NRSWA and must comply with the relevant duties and responsibilities imposed by NRSWA 1991, the Traffic Management Act 2004 and the associated Secondary Legislation and Codes of Practice.

1.3	It is the function of the Council as Highway Authority to monitor undertakers’ operations under NRSWA and ensure the rights of the highway user are protected.

1.4	Those Applicants not familiar with the requirements of NRSWA are strongly advised to appoint a contractor with the appropriate knowledge and accreditation to help complete the application and conduct the works on their behalf.

1.5	Significant financial penalties may arise to the Licensee for non-compliance with the relevant statutory duties or licence conditions.  Applicants should note that this liability cannot be delegated to any other person or organisation.

1.6	Officers in Network Management will provide advice and assistance to Applicants. Contact details are provided on the application form at Appendix A.

1.7	While the Council will give assistance, the legal responsibility to comply with NRSWA belongs to the Licensee and non-compliance can lead to prosecution. A Licensee may employ a sub-contractor to install the apparatus but again the legal responsibility remains with the Licensee.

1.8	If the application concerns a connection to a public sewer, separate permission is also required to do this from Yorkshire Water Plc.

1.9	If the works concerned require a road closure, a separate TTRO/N (Temporary Traffic Regulation Order/Notice) would be required, proof of approval should accompany this application.

1.10	Note that the process includes the following:

- Application to install and maintain private apparatus in the Highway.

- A requirement to consult statutory undertakers.

- The applicant must provide evidence of the appropriate public liability insurance cover.

- Provision of a suitable method statement.

- Submission of an Advance Notice Form .

- Submission of a Notice of Actual Start.

- Submission of a Reinstatement Form.

- A requirement on the licensee to ensure that Leeds City Council is made aware of any change to the ownership of the apparatus.

- Road closure approval if required.

- A requirement of the licensee to provide proof of adoption by the associated authority i.e. Yorkshire Water, Northern Power Grid, Northern Gas Networks etc. once the apparatus has been installed

2	PLANNING THE INSTALLATION

2.1	You should first of all complete the Street Works Licence Application form (Appendix 1) and send it to the Council at the address shown on the form. You will also need to supply the appropriate fee and 2 copies of a plan at scale 1:1250 or larger. This should clearly show the proposed location of the relevant apparatus.

2.2	The Council may require the Applicant to provide a cash bond to a pre-specified sum to secure performance of the reinstatement. The bond will be released on satisfactory completion of the guarantee period. You will be contacted should such a bond be required.

2.3	The Licensee must indemnify the Council against all and any claims made by a third party in connection with any aspects of the works.  This liability extends to the end of the guarantee period for the reinstatement.  The Licensee has an ongoing liability for the apparatus, should it fail and cause injury to another person or damage another person's property.

The Applicant must supply a copy of their current Public Liability Insurance Certificate with their application. This should provide a minimum level of cover of £10,000,000 for any one accident without limiting the number of claims arising out of any one incident.

2.4	One of the obligations placed on Licensees requires that the work be carried out by a street works accredited operative and supervisor.  The accredited supervisor has the specialist knowledge required to work on the highway and should be able to advise the licensee on the planning of the works.  Copies of the relevant accreditation cards must be supplied with the application, these must be for the type of surfaces being reinstated.

2.5	To comply with NRSWA the Licensee must take reasonable steps to avoid damage to another person’s apparatus which may be in the street.  Appendix 2 gives details of all the statutory undertakers who should be contacted. The Applicant is also required to make a declaration on the application form that they have contacted the relevant statutory undertakers and complied with their requirements.

2.6	Applicants should consult with the Council where details of the road reinstatement category will be provided. Advice will also be provided, if required, about reinstatement options, special requirements, restrictions on working hours, and any co-ordination with other planned works on highway.

To demonstrate that the works have been planned to be performed in a satisfactory manner the undertaker must prepare a method statement. A blank copy of the method statement is included at Appendix 3.

2.7	Applicants must provide a minimum of 20 working days’ notice from receipt of the application and/or all required information until the proposed start date of the works. This is to allow the Council time to send the statutory notice to the relevant authorities and to check and process the application. In certain circumstances longer notice periods will be required. The Applicant must complete the Advance Notice Form (Appendix 4) to specify this proposed start date and other relevant details. You will be informed of this once we receive your application, or you may contact us before making your application to discuss it.  If the work does not start on the agreed dates, the Applicant must forward a new Advance Notice Form.

2.8	Authorisation to commence the works will be given by the Council in writing once all the relevant information has been supplied to an acceptable standard. Work must not commence until this authorisation has been received. A checklist has been provided at Appendix 5 to assist Applicants in this.

3	COMPLETING THE WORKS

3.1	The works are to commence on the date specified on the Advance Notice Form submitted with the application. Within one working day of commencing work the Licensee must also submit to the Council the form ‘Notice of Actual Start’ (Appendix 6). The Council will inspect the site while work is in progress.

3.2	The Licensee is encouraged to ensure that the works are carried out to the timescale specified on the Advance Notice and Notice of Actual Start forms. Failure to do so could result in charges being applied by the Council under Section 74 of NRSWA. The current maximum charge is £10000 per day that the works over run.

3.3	Licensees are encouraged to carry out a permanent reinstatement of the highway immediately on completion of the works. Care must be taken to follow the requirements of the NRSWA Code of Practice, Specification for The Reinstatement of Highways particularly those relating to layer thicknesses and compaction. Please refer to the Code of Practice, and input the relevant details on Appendix 3, Application for Street Works Licence Method Statement.

3.4	The Licensee must advise the Council that the reinstatement has been completed, indicating whether to interim or permanent standard, by completing the Reinstatement Form (Appendix 7). This must be done within 24 hours of the completion date of the works. In the cases of interim reinstatement, a further form must be sent once the permanent reinstatement is completed, the permanent reinstatement must be completed within six months.

3.5	Failure to send any of the above-mentioned forms could result in the issue of a Fixed Penalty Notice or Prosecution in accordance with the current NRSWA Regulations.

3.6	The licensee is responsible for the trench reinstatement for a period of two years from submission of the Reinstatement Form for the permanent reinstatement (3 years if the trench is deeper than 1.5m).

3.7	The Council will inspect the trench on completion of the permanent reinstatement and again before the end of the guarantee period.  Any defects identified during these inspections must be remedied by the Licensee and defect inspection fees will be charged.

3.8	The Council may choose to take a core from the trench to ensure the correct depth of material has been used. If the core shows the reinstatement fails to comply with the NRSWA Code of Practice, Specification for The Reinstatement of Highways the Licensee will be required to carry out remedial works and will be charged the cost of taking the core and the defect inspection fees.

3.9	If the trench has to be repaired the guarantee period starts again.  The Licensee therefore saves considerable nuisance and expense by doing a good job the first-time round.

4	MAINTAINING THE APPARATUS

4.1	Licensees are responsible for maintenance of the apparatus for as long as it stays in their possession.

4.2	If the work is being carried out to existing apparatus for which there is no current street works licence the owner must apply for a licence.

4.3	If maintenance work is required which involves working on the highway the Licensee must notify the Council using the Advance Notice Form and must not proceed until written approval for the work to commence has been received from the Council. The Council will give guidance on the procedures to be followed.

4.4	The Licensee must notify the Council in writing prior to transferring the ownership of the apparatus or the properties which it serves.  They must also notify the person(s) to whom title of the apparatus or properties transfers, advising them of their obligations.

5	COSTS

  - Licence Fees

On submission of the relevant information the prospective Licensee is required to pay a licence fee of £560.30 (zero VAT). This payment covers the administration and legal costs in preparing the licence, an additional annual renewal fee will also apply, all in accordance with Schedule 3 of NRSWA. The fee also includes the current charges for inspection of up to 200m of trench. Additional inspection charges will apply if the works are proposed to be greater than 200m in length.

  - Bond

When specifically required the licensee will provide the Council with a cash bond to secure performance of the reinstatement. The bond will be released on satisfactory completion of the guarantee period.

  - Defect Inspection Fees

Following reinstatement of the works, if the Council identifies any defect the Licensee is required to pay a fee of £120 (zero VAT).  This covers inspections the Council carries out to ensure remedial works are satisfactorily completed.

  - Investigatory Works

Where a complaint from a member of the public requires a visit to the site and is found to result from a defect in a Licensee’s works, the Licensee is required to pay a fee of £68.00 (zero VAT), additional to any defect inspection fee.

Where the Council chooses to core a reinstatement, the licensee is required to pay the cost of the core, only if the core identifies an inadequate reinstatement.  This is additional to the defect inspection fee.

  - Emergency Work

If the Licensee is unable to respond to the need to carry out emergency work to make their site safe, the Council has the right to carry out this work and recharge the costs incurred.

  - Remedial Works

If the Licensee fails to carry out remedial works within 17 working days of being notified of a defect, the Council has the right to carry out further chargeable defect inspections and if necessary undertake the remedial work and recharge the costs involved.  This does not remove the Licensee’s responsibility for the reinstatement guarantee.

|  | Highways & Transportation Network Support Team 8th Floor Merrion House Merrion Way LS2 8DT |
| --- | --- |
|  | Contact: Tel: 0113 378 7007 raswa.forms@leeds.gov.uk |

Dear Sir/Madam

Permit Ref:

Location of works:

Dates of works:

*Please read this document carefully before signing*

Making a temporary excavation and the installation of apparatus in the Public Highway - Section 50 New Roads and Streetworks Act 1991.

- Leeds City Council operates a permit scheme on all parts of the publicly maintained highway.

- The permit scheme document can be viewed through following link: Street works permit scheme policy (leeds.gov.uk)

- Appropriate permits will be obtained through the permit scheme on behalf of the Applicant

The Applicant should read sections 12 & 14 of the permit scheme which explains the financial penalties associated with scheme and the Charges for Unreasonably Prolonged Occupation of the Highway

By signing this document you agree that

- The Applicant will complete the Advance Notice Form (Appendix 4) to specify a proposed works start date and other relevant details This must be submitted with the application.

- Confirmation of a proposed works start date will be provided by the Council following receipt of the Advance Notice Form.

- The Council will use the dates provided to put the permit into progress and close the permit down on behalf of the Applicant

- Should the agreed dates change, Applicants will be required to complete a further Advance Notice Form for submission to the Council prior to the original start date. If this is received later than the agreed start date then an administration fee will be applied. **Please note that in either case there is a risk that any new dates may not be agreeable**

Authorisation to commence the works will be given by the Council in writing following receipt of the Street Works Licence Agreement under Section 50 and Schedule 3 NRSWA, signed by both the Applicant and the Council acting as the Street Authority. Should works commence prior to authorisation being given by the Council, then the Licensee may incur financial penalties or prosecution in accordance with the Street works permit scheme policy (leeds.gov.uk)  sections 12 & 14

By signing this document you understand that

- Under Section 74 NRSWA, and in accordance with the Street works permit scheme policy (leeds.gov.uk)  sections 12 & 14, charges may be issued by the Council for the prolonged occupation of the highway should your work exceed the end date provided in your Advanced Notice form. The financial value being determined by the Reinstatement Category of the street and the type of works being undertaken. Should works be expected to extend beyond the end date agreed in the Applicant’s Advance Notice Form then agreement must be obtained to extend the duration before the agreed end date expires. A failure to achieve this may result in penalty charges being given by the Council (maximum of £10,000 per day).

- The Reinstatement Notice must be submitted (Appendix 7 of the application form) once the reinstatement is complete detailing closing dimensions to raswa.forms@leeds.gov.uk.

If you have any questions regarding the above process and potential charges please contact raswa.forms@leeds.gov.uk FAO Senior Engineers prior to signing below

Name  _______________________

Signed  ______________________

Dated  _______________________

Yours faithfully,

A Hodgson

Traffic Manager

Appendix 1

NEW ROADS AND STREET WORKS ACT 1991

SECTION 50

STREET WORKS LICENCE APPLICATION FOR PLACING OR RETAINING APPARATUS IN THE HIGHWAY

SECTION 1 - OWNER OF APPARATUS

Surname:				Forenames:

Or full registered name of company:

Address:

Postcode:

Tel: 		Mobile:

Email address:

Proof of adoption: Please attach proof the apparatus is to be adopted by the relevant utility

___________________________________________________________________________

SECTION 2 – LOCATION OF PROPOSED WORKS

Location:

Address:

Postcode:

___________________________________________________________________________

SECTION 3 - DETAILS OF PERSON OR COMPANY CONDUCTING THE WORKS

Name:

Address:

Tel No:

The work will be supervised by the following accredited supervisor:-

Name:

Registration number:

_______________________________________________________________

Note: Copies of accreditation for supervisor and operatives must also be supplied

SECTION 4 - INSURANCE

I hereby agree to indemnify the Council against all and any claims made by a third party in connection with any aspects of the works.  I understand that this liability extends to the end of the guarantee period for the reinstatement and includes an ongoing liability for the apparatus, should it fail and cause injury to another person or damage another person's property.

I enclose a copy of the public liability insurance certificate

___________________________________________________________________________

SECTION 5 - DECLARATION BY OWNER OF APPARATUS

I confirm that the foregoing details are correct and acknowledge that the works referred to above must be conducted in accordance with the requirements of the New Roads and Street Works Act 1991 and associated legislation and codes of practice, together with any conditions imposed in the relevant licence.

I also acknowledge the statutory need for me to pay the prescribed inspection fees which will be imposed, including any defect inspection fees and the cost of any necessary remedial works conducted by Leeds City Council during the guarantee period, along with the required administration / capitalised annual charge fees.

I also acknowledge that I have read and understood the Notes for Guidance.

I have contacted the relevant statutory undertakers in Appendix 2 and have obtained details of other apparatus which may be affected by my works.

I confirm that a payment of £560.30 will be made prior to works being carried out. £516.30 being the total of the required licence fee together with the inspection fee for routine inspections to be carried out by the Council.

Signed: 		            Dated:

Print Name:			Position:

(Street Works Licence Applicant)

Please return via email to:

Contact: Network Support Team Tel: 0113 3787007 Email: raswa.forms@leeds.gov.uk Highways & Transportation Network Support Team Leeds City Council 8th Floor, Merrion House 110 Merrion Way Leeds LS2 8BB.

(Appendix 2)

STATUTORY UNDERTAKERS REQUIREMENTS

To comply with Section 69 of NRSWA 1991 Act where works are likely to affect another person's apparatus in the street, licensees must take all reasonable practicable steps:-

a)	to give the person to whom the apparatus belongs reasonable facilities for monitoring the execution of the works, and

b)	to comply with any requirements made which are reasonably necessary for the protection of apparatus or securing access to it.

All statutory undertakers must therefore be contacted before the commencement of any work in the highway.

# NRSWA 1991 – Schedule of Undertakers

This schedule is continually reviewed as new undertakers commence installations within Leeds, company contact details alter and companies change name due to mergers etc.

The Schedule was last updated on: January 2017

The schedule is in two parts.

Part 1 lists all undertakers who have installed networks across the whole of Leeds. All these undertakers should be consulted under normal circumstances.

Part 2 lists undertakers who need only be consulted if an assessment of a specific site identifies they may be affected. They either

- have apparatus in only a specified part of the city or

- have apparatus, all of which can be identified through a site survey (e.g.             	servicing telephone kiosks etc)

Part 1: Undertakers who have installed networks across the whole of Leeds.

| Yorkshire Water |  | Undertaker Code: QB01-QB99 |  |  |
| --- | --- | --- | --- | --- |
| Address Telephone Fax Email Website Comments | Yormap, PO Box 682, Bradford, BD3 7YT 01274 692334 01274 253502 safemove@yorkshirewater.com www.safe-move.co.uk Enclose separate requests for water supply & sewerage |  |  |  |
| Northern Gas Networks |  | Undertaker Code: XX03 |  |  |
| Address Telephone Email Website Comments | Plant Protection, 1st Floor, 1 Emperor Way, Doxford International Business Park, Sunderland, SR3 3XR 0845 6340508 (Dial before you Dig option 6, 09:00-17:00 Monday to Friday) plantprotection@northerngas.co.uk www.northerngasnetworks.co.uk Plans available on CD, supplied on request |  |  |  |
| Northern Powergrid (YEDL / CE Electric) |  | Undertaker Code: QA34/44 |  |  |
| Address Telephone Fax Email Website | Records Information Centre, New York Road, Shiremoor, Newcastle Upon Tyne, NE27 0LP 0191 229 4294 / 4295 0191 229 4287 safediggingplans@northernpowergrid.com www.Safedig.co.uk |  |  |  |
| Openreach |  | Undertaker Code: BC02/04 |  |  |
| Address Telephone Email Wesbite Maps by email Comments | Notice Handling Centre, PP 404B, Telecom House, Trinity Street, Hanley, Stoke on Trent, ST1 5ND Dial Before You Dig - 0800023 2023 opt 5 opt 1 nnhc@openreach.co.uk www.dialbeforeyoudig.com (07:30-18:00 Monday to Friday) We will only accept mark up requests via e mail (include MARK UP in the title) using the attached form. L:\RASWA\RASW-ADM\RASWA FORMS\Openreach Mark Up Request Form.xls |  |  |  |
| Virgin Media |  |  | Undertaker Code: NK100 |  |
| Address Telephone Fax Email Comment | National Plant Enquiries, Unit 1A, Scimitar Park, Courtauld Road, Basildon, SS13 1ND 0870 888 3116/0800 408 0088 01268 468557 plant.enquiries.team@virginmedia.co.uk Includes Telewest. Yorkshire Cable were a subsidiary of Telewest Broadband and have a cable in the south of the city For Diversionary works/new developments, please contact Virgin Media, 1 Dove Wynd, Strathclyde Business Park, Bellshill, ML4 3AL |  |  |  |
| National Grid (Electricity) |  |  | Undertaker Code: HP01 |  |
| Address Telephone Email Website | National Grid Plant Protection Team, National Grid, Block 1, Floor 2, Brick Kiln Street, Hinckley, LE10 0NA 0800 688588 (Mon – Fri 0800 to 1630) plantprotection@nationalgrid.com www.beforeyoudig.nationalgrid.com |  |  |  |
| Vodafone |  |  | Undertaker Code: QL10 |  |
| Address Telephone Email Website Comments | Vodafone Plant Enquiries, c/o Atkins, The Hub, 500 Park Avenue, Aztec West, Almondsbury, BS32 4RZ 01454 662881 osm.enquiries@atkinsglobal.com http://www.candwplantenquiries.com C&W became Vodafone on 1 April 2013. The Vodafone: Fixed network covers Cable & Wireless UK, Energis Communications Limited, Thus Group Holdings Plc, Mercury and Your Communications Limited |  |  |  |
| CityFibre |  |  | Undertaker Code: KG01 |  |
| Address Telephone Email Website Comments | CityFibre Holdings Limited, Rutherford House, Warrington Road, Birchwood, Warrington, WA3 6ZH 0333 150 7282 plant.enquiries@cityfibre.com www.cityfibre.com Old Torch Network, Kingston Communications/Affiniti (MY01) and CityFibre |  |  |  |
| BSkyB Telecommunications Ltd |  |  | Undertaker Code: ZG01 |  |
| Address Telephone Fax Email Wesbite Comment | NRSWA Department, Network Infrastructure and Planning, 70 Buckingham Avenue, Slough, Berkshire, SL1 4PN 0207 0323234/250 0207 0323252 nrswa@sns.bskyb.com/nrswa@bskyb.com www.easynet.com Formerly Ipsaris and Fibreway Ltd |  |  |  |
| *Gamma Telecom |  |  | Undertaker Code: YB01 |  |
| Address Telephone Fax Contact Name Email Comments | Unit C, Focal Point, 2nd Avenue, Trafford Park, Manchester, M17 1FG 0333 240 3451 or 24hr NOC on 0845 241 6000 0161 8775704 Mark Worden Please use LSBUD service or you can email http://www.linesearchbeforeudig.co.uk Purchased the National Fibre Network from Atlantic when company went into receivership |  |  |  |
| *Verizon Business |  |  | Undertaker Code: MK01 |  |
| Address Email Fax Comments | Plant Protection Officer, 2-6 St Pancras Way, London, NW1 0QG osp-team@uk.verizon.com 01293 553871 Includes MCI WorldCom, MFS |  |  |  |
| *Geo Network |  |  | Undertaker Code: YM12 |  |
| Address Website Comment | BT plc (Geo), Hampstead TE, PP G25, 361 Finchley Road, London, NW3 6EX www.geo-uk.net/www.linesearch.org 186k & Hutchinson 3G |  |  |  |
| Interoute Communications Limited (Vtesse) |  |  |  | Undertaker Code: |
| Address | John Tate Road, Foxholes Business Park, Hertford, SG13 7DT |  |  |  |
| Telephone Website | 01992 532151 https://vtplant.vtesse.com |  |  |  |
| Email | plant.enquiries@vtesse.com |  |  |  |
| Comment | Nominal charge for plant enquiries |  |  |  |

Part 2 - Lists undertakers who need only be consulted if an assessment of a specific site identifies they may be affected

| COLT Telecomms (City of London Telecomms Ltd) |  | Undertaker Code: CS31 |
| --- | --- | --- |
| Address Telephone Fax Contact Name Email Address Website Comments | Colt Plant Protection Centre, c/o McNicholas Construction (Colt Managing Agents), 117 Bushey Mill Lane, Walford, Herts, WD24 7UN 020 82366612 020 82366600 John Richardson inbound@colt.net www.colt.net Currently in South of city only |  |
| *Royal Mail |  | Undertaker Code: SW01 |
| Address Telephone Comments | Leeds Mail Centre, Leodis House, Leodis Way, Stourton, Leeds, LS10 1AZ 0113 3866658 Royal Mail post pouches and boxes. Consult only where apparatus is evident |  |
| ES Pipelines Address Hazledon, Station Road, Leatherhead, Surrey, KT22 7AA Telephone 01372 227560 |  | Undertaker Code: ZY001 |
| Email Comments | plant@espipelines.com Formerly British Gas Connections |  |
| *FibreSpan Ltd* |  | Undertaker Code: XM01 |
| Address Telephone Fax Email Wesbite Comments | The Quay, Channel Way, Ocean Village, Southampton, SO14 3QG 0800 6928900 0845 3138901 info@fibrespan.co.uk www.fibrespan.com Have installed apparatus in Woodhouse, Burley, Armley & Central Leeds |  |
| *Infolines Public Networks Ltd* |  | Undertaker Code: RE01 |
| Address Telephone Fax Email | The White House, Lichfield Street, Tamworth, B79 7QF 01827 262300 01827 55987 (5 digits is correct) service@interphonenetworks.co.uk |  |
| METRO |  | Undertaker Code: MO01 |
| Address Comments | West Yorkshire PTE, Property Services, Crow Nest Lane, Elland Road, Beeston, Leeds, LS11 8AT Consult only where apparatus is evident. |  |
| SSE Mapping Services |  | Undertaker Code: YE01 |
| Address Telephone Fax Contact Name Email Comments | SSE Mapping Services, PO Box 6206, Basingstoke, Hampshire, RG24 8BW 01256 337294 01256 337295 David Green mapping.services@sse.com telecom.alterations@sse.com Neos Network is the telecoms business of Scottish and Southern Electric plc (SSE). One cable only, Gelderd Road, Ring Road Beeston, Royds Lane, Whitehall Road. Previous company name was Neos Networks Ltd |  |
| Arcavia - NWP Spectrum Ltd. / Spectrum Interactive plc |  | Undertaker Code: NW01 |
| Address Telephone Fax Contact Name Comments | PO Box 668, Parker House, Maylands Avenue, Hemel Avenue, Hemel Hempstead, Hertfordshire, HP2 4WA 01442 205500 01442 261001 Robert Burton City Centre payphone installation. Consult only where apparatus is evident. Previously called New World Payphone. |  |
| Instalcom Ltd |  | Undertaker Code: NW01 |
| Address Telephone Fax Email Website Comments | Instalcom, Unit 1, Borehamwood Industrial Park, Rowley lane, Borehamwood, Hertfordshire, WD6 5PZ 020 8731 4600 020 8731 4601 plantenquiries@instalcom.co.uk www.instalcom.co.uk Global Crossing & Fibernet, Racal Telecom (South of City) |  |
| TeliaSonera |  | Undertaker Code: ZM01 |
| Address Telephone Email | Telent, Teliasonera Line Plant Enquiries, Crompton Close, Basildon, Essex, SS14 3BA 01268 595608 telenttelia.plantenquiries@telent.com |  |
| GTC pipelines |  |  |
| Address Telephone Website Comments | GTC Energy House, Woolpit Business Park, Woolpit, Bury Saint Edmonds, IP30 9UP 0845 055 6199 (EXT 4) pe.gtc-uk.co.uk/Plant_Enquiry Includes: Independent Pipelines Limited, Quadrant pipelines, Electricity Network Company, Independent Power Networks, Utility Grid Installations |  |
| O2 |  | Undertaker Code: MG01/02 |
| Address Telephone Fax Wesbite | o2 (UK Ltd), 260 Bath Road, Slough, SL1 4DX 01753 565 003 01753 565 010 www.o2.co.uk |  |
| Turner & Townsend (previously Orange PCS) |  | Undertaker Code: RT01 |
| Address Telephone Email | 63 Church Street, Birmingham, B3 2DP 0121 2621100 mbnlplantenquiries@turntown.com |  |

(Appendix 3)

Application for Street Works Licence

Description of works:

Description of Traffic and Pedestrian Management:

(Please note that the use of temporary road closures/traffic signals will require a separate application, a separate TM plan may be required for complex situations).

Proposed Working Dates and Times: (Including weekends if applicable).

Road Reinstatement Category :

Proposed Reinstatement Details: (materials, thickness etc)

Consultation carried out:  (e.g. Bus operators, residents, emergency services, etc.)

(Appendix 4)

Advance Notice Form

To:

Contact: Network Support Team Tel: 0113 3787007 Email: raswa.forms@leeds.gov.uk Highways & Transportation Network Support Team Leeds City Council 8th Floor, Merrion House 110 Merrion Way Leeds LS2 8BB

From: (Insert your address)

Tel ………………………………………………

Out of hours tel ………………………………………………

Email Address ………………………………………………

Proposed Start Date ………………………………………………

Proposed End Date ………………………………………………

Street Name ………………………………………………

Locality/District ………………………………………………

Location of Work (e/g o/s 1) ………………………………………………

Works Located in	Carriageway     Footway      Verge

Average Depth of Excavation ………………………………………………

(Appendix 5)

Checklist of Information to be supplied by Applicants

□ 	Completed Application Form (Appendix 1)

□	Licence fee (Please contact Network Management to arrange payment)

□	Two copies of apparatus location plan (min 1:1250)

□	TM plan

□	Bond fee if applicable

□	Copy of public liability insurance certificate (At least £10 million)

□	Copy of supervisor and operative NRSWA accreditation cards (both sides) must cover the type of surfaces being reinstated.

□	Copy of method statement (Appendix 3)

□	Advance notice form (Appendix 4)

Checklist of information to be supplied once works have commenced

□	Notice of actual start form (Appendix 6)

□	Reinstatement form (Appendix 7)

(Appendix 6)

Notice of Actual Start Form

To:

Contact: Network Support Team Tel: 0113 3787007 Email: raswa.forms@leeds.gov.uk Highways & Transportation Network Support Team Leeds City Council 8th Floor, Merrion House 110 Merrion Way Leeds LS2 8BB

From:	(Insert your address)

Telephone Number ………………………………..

Out of hours telephone number ………………………………..

Email Address ………………………………………….

Works Reference number (supplied on approval letter) …………………………….

Actual Start Date ………………………………….

Proposed End Date ……………………………….

Actual Start Time ………………………………….

Proposed End Time ………………………………….

Street Name …………………………………………………………………………….

Locality/District …………………………………………………………………………..

Location of Work (e/g o/s 1) ………………………………………………………......

Works Located in:	Carriageway □     	Footway □    Verge □

Average Depth of Excavation …………………………………………………………

(Appendix 7)

Reinstatement Notice Form

To:

Contact: Network Support Team Tel: 0113 3787007 Email: raswa.forms@leeds.gov.uk Highways & Transportation Network Support Team Leeds City Council 8th Floor, Merrion House 110 Merrion Way Leeds LS2 8BB

From:	(Insert your address)

Telephone number ……………………………………………..

Out of hours telephone number ………………………………..

Email Address ……………………………………………………..

Works Reference number (supplied on approval letter) …………………………….

Actual Start Date ………………………………….

Works End Date …………………………………..

Street Name …………………………………………………………………………….

Locality/District …………………………………………………………………………..

Location of Work (e/g o/s 1) ……………………………………………………......

Depth and Dimension(s) of Excavation(s)

Site 1 ………………………………………….C/W-F/W-Verge   Interim-Permanent

Site 2 ………………………………………… C/W-F/W-Verge   Interim-Permanent

Site 3 ………………………………………… C/W-F/W-Verge   Interim-Permanent

(Circle the above as appropriate)